## Implementation Guide: Distributed Federated Learning for Spectrum Sensing

Implementing the system described in the paper by Yi Shi, Yalin E. Sagduyu, and Tugba Erpek involves simulating a decentralized wireless sensor network (WSN) where nodes collaboratively train a Feedforward Neural Network (FNN) for signal classification (e.g., distinguishing QPSK from BPSK) using Federated Averaging ($FedAvg$), without sharing raw I/O data.

---

### Step 1: Environment & Dataset Setup

* **Simulator Framework:** Use Python with network/graph libraries (`networkx`) for topology generation, and deep learning libraries (`PyTorch` or `TensorFlow/Keras`) for the FNN model.

* **Data Generation (I/Q Data & Features):**

* Simulate a transmitter at coordinates $(0, 0)$ generating **BPSK** and **QPSK** signals.

* Extract features from raw I/Q samples: compute **phase shift** and **power level**.

* Format each sample to have 32 features (e.g., 16 bits per sample or two sets of 2-bit phase/power calculations for QPSK).

* **Sensor Deployment:** Place sensors in a designated area (e.g., $[100, 1000] \times [100, 1000]$) with a defined communication range (e.g., $400$ units).

---

### Step 2: Define Network Topologies

Implement graph adjacency matrices for the different topologies evaluated in the paper using `networkx`:

* **Line / Ring:** Nodes connected sequentially or in a closed loop (degree = 2).

* **Star:** A central node connected to $N-1$ peripheral nodes. *(Note: Remember that unlike client-server architectures, the center node's model isn't globally forced onto others instantly).*

* **Grid:** A 2D lattice where most interior nodes have up to 4 neighbors (left, right, up, down).

* **Random Network:** Nodes randomly distributed where edges are created if the euclidean distance between two nodes is $\le 400$.

---

### Step 3: Configure the Local Deep Learning Model (FNN)

Set up the Feedforward Neural Network at each sensor node based on the paper's specifications:

* **Input Size:** 32 features.

* **Hidden Layers:** 3 dense layers with sizes **[128, 64, 32]** and **ReLU** activation.

* **Output Layer:** Softmax activation (size 2 for BPSK vs. QPSK classification).

* **Hyperparameters:**

* Optimizer: **RMSprop**

* Loss Function: **Categorical Cross-Entropy**

* Dropout Rate: **0.2**

* Total Parameters: ~14,626 weights.

---

### Step 4: Implement the Distributed Training Loop

For each training round $t = 1, 2, \dots, T$:

1. **Local Training:** Each sensor $i$ trains its current local FNN model $w_i^{(t)}$ using its locally sensed dataset for a few local epochs.

2. **Neighbor Broadcasting:** Each sensor broadcasts its updated model parameters $w_i^{(t)}$ to its direct graph neighbors. Introduce a **broadcast probability** (e.g., $100\%$, $50\%$, $25\%$, or $10\%$) to simulate energy-saving probabilistic participation.

3. **Packet Loss Emulation:** Randomly drop model update packets based on a defined loss probability ($0\%$, $5\%$, $20\%$, $50\%$) to test wireless robustness.

4. **Federated Averaging ($FedAvg$):** Upon receiving models from available neighbors, each sensor computes its new model for the next round by averaging its own model weights with the received valid neighbor weights:

$$w_i^{(t+1)} = \frac{1}{\vert{}\mathcal{N}_i^\prime\vert{} + 1} \left( w_i^{(t)} + \sum_{j \in \mathcal{N}_i^\prime} w_j^{(t)} \right)$$

where $\mathcal{N}_i^\prime$ represents the set of successfully received neighbor models.

---

### Step 5: Evaluation & Convergence Tracking

* **Global Test Dataset:** Evaluate models against a centralized test set containing data samples from all sensor locations to ensure generalized accuracy.

* **Convergence Criterion:** Track the best average accuracy across rounds. Declare convergence when improvements stay below a small threshold ($\epsilon = 0.01$) for continuous rounds ($M = 100$).

* **Metrics to Plot:**

* Average accuracy vs. Time (Rounds).

* Impact of network topologies on convergence speed.

* Trade-offs between lower broadcast probabilities (energy savings) and signal classification accuracy.

---

Would you like a Python code skeleton using PyTorch and NetworkX to kick off the simulation framework for one of these topologies (like the Grid or Random network)?





To understand how raw radio frequency (RF) data turns into those 32 features, it helps to break down what **I/O (In-phase and Quadrature) data** is and how wireless signals are processed.

---

### Step 1: What is Raw I/Q Data?

When a wireless sensor (like an SDR or a spectrum monitoring receiver) listens to the airwaves, it doesn't hear audio or see text. Instead, it measures radio waves as continuous streams of complex numbers called **I/Q data**:

* **I (In-phase):** Represents the cosine (real) component of the wave.

* **Q (Quadrature):** Represents the sine (imaginary) component of the wave.

Every single "sample" captured by the sensor is a pair of numbers: $(I_k, Q_k)$.

---

### Step 2: How Do We Get Features from I/Q Data?

Instead of feeding raw $I$ and $Q$ numbers directly into a neural network, wireless communication systems extract physical characteristics of the signal—specifically **phase shift** and **power level**—over a block of bits.

As mentioned in the paper:

1. **Bit Grouping:** The system looks at a block of **16 bits** of data at a time as one sample.

2. **Modulation Types (BPSK vs. QPSK):**

* **BPSK (Binary Phase Shift Keying):** Encodes data by shifting the phase of the wave between 2 states ($0^\circ$ or $180^\circ$).

* **QPSK (Quadrature Phase Shift Keying):** Encodes data by shifting the phase into 4 states ($45^\circ, 135^\circ, 225^\circ, 315^\circ$).

---

### Step 3: Why Exactly 32 Features?

The paper states: *"We consider features for 16 bits as one sample. For BPSK, there are 32 features. For QPSK, we calculate two sets of phase shift and power level for 2 bits so that the number of features in a sample is also 32."*

Here is the exact math of how 16 bits translate into a **32-feature vector** for your neural network input:

#### Case A: BPSK Signals

* You are looking at a block of **16 bits**.

* For each bit (or small sub-symbol chunk), the receiver calculates 2 physical metrics:

1. **Phase Shift** (how much the wave shifted)

2. **Power Level** (how strong the signal is)

* Since there are 16 bits, and you extract 2 metrics per bit, you get:

$$16 \text{ bits} \times 2 \text{ metrics (Phase \& Power)} = \mathbf{32 \text{ features}}$$

#### Case B: QPSK Signals

* QPSK carries 2 bits per symbol. Therefore, 16 bits translate into 8 symbols.

* The paper notes that it calculates **two sets** of phase shift and power level metrics for every 2-bit chunk.

* Mathematically:

$$8 \text{ symbols} \times 4 \text{ metrics per symbol} = \mathbf{32 \text{ features}}$$

By structuring it this way, **both BPSK and QPSK samples end up with exactly 32 features**, which allows them to be fed into the exact same input layer of your Feedforward Neural Network (FNN).

---

### Step 4: Python Code Example of Feature Extraction

To visualize how this looks in code, here is a simplified simulation of turning raw I/Q values into those 32 features:

```python

import numpy as np

def extract_features_from_iq(i_samples, q_samples):

"""Takes raw I and Q arrays and converts them into 32 features

representing power level and phase shift for a 16-bit block.

"""

# 1. Calculate Power Level: P = I^2 + Q^2

power_levels = (i_samples**2) + (q_samples**2)

# 2. Calculate Phase Shift: Theta = arctan2(Q, I)

phase_shifts = np.arctan2(q_samples, i_samples)

# Combine them into a single array

combined_features = np.concatenate([power_levels, phase_shifts])

# Ensure the final output is exactly 32 features

# (e.g., 16 power values + 16 phase values = 32)

return combined_features[:32]

# --- Example Test ---

# Simulate 16 raw I and Q readings from a sensor

mock_i = np.random.randn(16)

mock_q = np.random.randn(16)

feature_vector = extract_features_from_iq(mock_i, mock_q)

print("Shape of feature vector:", feature_vector.shape)  # Output: (32,)

```

Does this breakdown make it clearer how raw radio signals are converted into the 32-element input vector required by the neural network?