To accurately classify signal strength into categories like **Excellent, Good, Poor, or Bad** using a 32-input feature vector in a federated learning network (such as Radio 15 evaluating its links to Radios 14 and 16), the features must capture both absolute signal power and environmental noise quality.

The most effective set of 32 input features combines **Received Power, Signal-to-Noise metrics, and statistical I/Q properties**:

---

### The Optimal 32-Feature Input Vector Breakdown

To fill the 32 inputs required by the FNN model (matching the neural network's input layer), you can structure the feature vector into four distinct categories:

#### 1. Power Metrics (Features 1–8)

These measure how loud or strong the radio wave is when it reaches the receiver.

* **RSSI (Received Signal Strength Indicator):** Absolute power level received from the neighboring radio.

* **Mean Power & Peak Power:** Average and maximum power calculated over the sample window.

* **Power Variance:** Fluctuations in signal power over time (helps detect fading).

#### 2. Signal Quality & Interference Metrics (Features 9–16)

Power alone doesn't mean a signal is good; it needs to be clear of static and interference.

* **SNR (Signal-to-Noise Ratio) Proxy:** The ratio between the signal power and the background noise floor.

* **SINR (Signal-to-Interference-plus-Noise Ratio):** Measures how much unwanted interference is present from other signals.

* **Noise Floor Estimation:** The baseline ambient energy level when no transmission is active.

#### 3. Statistical Moments of I and Q (Features 17–24)

Raw I/Q samples contain hidden distribution details about signal distortion.

* **Standard Deviation of I and Q:** Measures the spread of the complex waveform.

* **Skewness of I and Q:** Measures asymmetry in the signal distribution (indicates hardware clipping or non-linear distortion).

* **Kurtosis of I and Q:** Measures "tailedness" or peakiness of the signal, which helps identify severe multi-path fading or impulse noise.

#### 4. Channel Stability & Envelope Metrics (Features 25–32)

These capture how stable the link is between Radio 15 and its neighbors (Radios 14 and 16) over consecutive temporal blocks.

* **Envelope Variance:** Rate of change in the signal envelope.

* **Root Mean Square (RMS) Delay Spread:** Measures signal smearing caused by reflections.

* **Padding/Windowed Averages:** Rolling statistical averages across sub-blocks to complete the exact count of 32 features.

---

### How Radio 15 Uses This with Radios 14 and 16

1. **Local Measurement (Radio 15):** When Radio 15 receives packets from Radio 14 and Radio 16, it extracts these 32 features from the incoming preambles or pilot tones.

2. **Local Classification:** Radio 15 feeds this 32-feature vector into its local FNN model to classify the link condition (e.g., classifying the link to Radio 14 as *Good* and Radio 16 as *Poor*).

3. **Federated Averaging:** Radio 15 uploads its *model weights* (not the raw features or data) to its neighbors or the network graph.

4. **Central Server Aggregation:** If a central monitoring server is connected to the graph, it collects performance summaries or weights from all 32 nodes. By compiling these reports, the server maps the **global range** of signal strength across the entire topology (e.g., identifying that the network's weakest link drops to $-105\text{ dBm}$ while the strongest peaks at $-50\text{ dBm}$).




import numpy as np

def generate_synthetic_signal_data(num_samples_per_class=250):

"""Generates synthetic 32-feature vectors for 4 signal strength classes:

0: Excellent, 1: Good, 2: Poor, 3: Bad

"""

features_list = []

labels_list = []

# Define characteristic power centers (in arbitrary scaled units or dBm proxies)

profiles = {

0: {"power_mean": -55.0, "noise": 0.1, "label": 0},  # Excellent

1: {"power_mean": -75.0, "noise": 0.3, "label": 1},  # Good

2: {"power_mean": -95.0, "noise": 0.7, "label": 2},  # Poor

3: {"power_mean": -110.0, "noise": 1.5, "label": 3} # Bad

}

for class_id, profile in profiles.items():

for _ in range(num_samples_per_class):

# Generate 32 features based on the signal profile with random variance

power_metrics = np.random.normal(profile["power_mean"], 3.0, 8)

quality_metrics = np.random.exponential(profile["noise"], 8)

statistical_moments = np.random.uniform(-1.0, 1.0, 8)

channel_stability = np.random.normal(0.5, profile["noise"], 8)

feature_vector = np.concatenate([

power_metrics, quality_metrics, statistical_moments, channel_stability

])

features_list.append(feature_vector)

labels_list.append(class_id)

return np.array(features_list), np.array(labels_list)

# --- Test the Generator ---

X, y = generate_synthetic_signal_data(num_samples_per_class=10)

print("Generated Features Shape:", X.shape) # Expected: (40, 32)

print("Generated Labels Shape:", y.shape)   # Expected: (40,)

import copy

import numpy as np

import torch

import torch.nn as nn

import torch.optim as optim

# --- 1. Define the FNN Model for Signal Strength Classification ---

class SignalStrengthFNN(nn.Module):

def __init__(self):

super(SignalStrengthFNN, self).__init__()

self.fc1 = nn.Linear(32, 128)

self.fc2 = nn.Linear(128, 64)

self.fc3 = nn.Linear(64, 32)

self.out = nn.Linear(32, 4)  # 4 classes: Excellent, Good, Poor, Bad

self.relu = nn.ReLU()

self.dropout = nn.Dropout(0.2)

self.softmax = nn.Softmax(dim=1)

def forward(self, x):

x = self.relu(self.fc1(x))

x = self.dropout(x)

x = self.relu(self.fc2(x))

x = self.dropout(x)

x = self.relu(self.fc3(x))

return self.softmax(self.out(x))

# --- 2. Synthetic Data Generator ---

def generate_node_data(dominant_class, num_samples=100):

"""Generates synthetic 32-feature samples skewed toward a specific signal condition."""

profiles = {

0: {"power_mean": -55.0, "noise": 0.1},  # Excellent

1: {"power_mean": -75.0, "noise": 0.3},  # Good

2: {"power_mean": -95.0, "noise": 0.7},  # Poor

3: {"power_mean": -110.0, "noise": 1.5}  # Bad

}

features_list, labels_list = [], []

profile = profiles[dominant_class]

for _ in range(num_samples):

power_metrics = np.random.normal(profile["power_mean"], 3.0, 8)

quality_metrics = np.random.exponential(profile["noise"], 8)

statistical_moments = np.random.uniform(-1.0, 1.0, 8)

channel_stability = np.random.normal(0.5, profile["noise"], 8)

feature_vector = np.concatenate([

power_metrics, quality_metrics, statistical_moments, channel_stability

])

features_list.append(feature_vector)

labels_list.append(dominant_class)

return torch.tensor(np.array(features_list), dtype=torch.float32), torch.tensor(np.array(labels_list), dtype=torch.long)

# --- 3. Federated Averaging Function ---

def federated_averaging(local_state_dicts):

"""Averages model weight dictionaries from Radio 15, 14, and 16."""

global_dict = copy.deepcopy(local_state_dicts[0])

for key in global_dict.keys():

for i in range(1, len(local_state_dicts)):

global_dict[key] += local_state_dicts[i][key]

global_dict[key] = global_dict[key] / float(len(local_state_dicts))

return global_dict

# --- 4. Main Simulation Execution ---

if __name__ == "__main__":

print("Initializing Radio Neighborhood (Radios 14, 15, and 16)...")

# Instantiate models for each radio

radio_models = {

14: SignalStrengthFNN(),

15: SignalStrengthFNN(),

16: SignalStrengthFNN()

}

# Give each radio unique local data (Non-IID simulation)

# Radio 14 experiences 'Good' signals (Class 1)

# Radio 15 experiences 'Poor' signals (Class 2)

# Radio 16 experiences 'Excellent' signals (Class 0)

radio_data = {

14: generate_node_data(dominant_class=1, num_samples=50),

15: generate_node_data(dominant_class=2, num_samples=50),

16: generate_node_data(dominant_class=0, num_samples=50)

}

optimizers = {

node: optim.RMSprop(model.parameters(), lr=0.01)

for node, model in radio_models.items()

}

criterion = nn.CrossEntropyLoss()

num_rounds = 3

print(f"\n--- Starting Distributed Federated Learning for {num_rounds} Rounds ---")

for round_idx in range(num_rounds):

print(f"\n[Round {round_idx + 1}] Local Training Phase...")

# Step A: Each radio trains locally on its own private data

for node in [14, 15, 16]:

model = radio_models[node]

optimizer = optimizers[node]

X_train, y_train = radio_data[node]

model.train()

optimizer.zero_grad()

outputs = model(X_train)

loss = criterion(outputs, y_train)

loss.backward()

optimizer.step()

print(f"Radio {node} completed local training. Loss: {loss.item():.4f}")

print(f"\n[Round {round_idx + 1}] Neighbor Exchange & Aggregation Phase for Radio 15...")

# Step B: Radio 15 collects models from its direct neighbors (14 and 16)

# In a real network, these are sent via wireless packets. Here we pass state dicts.

neighbor_models = [

radio_models[15].state_dict(),  # Radio 15's own model

radio_models[14].state_dict(),  # Radio 14's model

radio_models[16].state_dict()   # Radio 16's model

]

# Step C: Radio 15 performs Federated Averaging

averaged_weights = federated_averaging(neighbor_models)

# Radio 15 updates its model with the collaborative neighborhood knowledge

radio_models[15].load_state_dict(averaged_weights)

print("Radio 15 successfully aggregated weights from Radios 14, 15, and 16!")

print("\nSimulation complete! Radio 15 has successfully blended local and neighbor signal profiles via Federated Learning.")